Sampleprepp: proteinextraction, assey and standardise to 2 ug/ul protein.

Add loading buffer to dilute samples to 1.5 ug/ul

- Loading buffer test:

Pool protein from several samples to create a representable average of all samples to test loading buffers and antibodies.

Compare 4 different loading strategies:

- SDS-urea w/inhibitors and tiny amount of bromophenol blue @ room temp

- SDS-urea w/inhibitors and tiny amount of bromophenol blue @70 celcius for 10 min

- Laemmli loading buffer @ 70 celcius for 10 min

- Laemmli loading buffer @ 95 celcius for 5 min

SDS-PAGE:

- Place precast gel in rack with running buffer

Running buffer (1 Liter):

- 100 ml 10X Tris-Glycine buffer

- 10 ml of 10% SDS

- Add ddH2O to final volume 1000ml

10X Tris-Glycine runnning buffer (1 Liter):

- 30.3g Tris (MW 121.14) (final concentration 250 mM, 25 mM in 1X solution)

- 144.1g Glycine (MW 75.07) (Final concentration 1.92 M, 192 mM in 1X solution)

- 10g SDS

- Add ddH2O to final volume 1000ml

- Load ~15ug per lane + ladder

- Run gel 250V for ~1h  (untill blue line exits the gel) @ RT on icebath

Protein transfer to PVDF membrane:

(cutPVDF membrane to 8.5 cm x 14 cm, don’t touch white membrane!!! , actvate PVDF membrane by incubating 5 min in MeOH, and in transfer buffer for 5 min)

- Run transfer @400 mA for 1h @ 4 celcius

Protein visualization on PVDF membrane using MemCode:

(Keep PVDF membrane side that was facing the Gel during transfer up at all times, and do not let PVDF membrane dry!

- Photograph the Stained PVDF membrane in G:BOX on clear plastic sheet

- Cut PVDF membrane into desirable strips (vertically and horizontally at desirable kDa)

Protein identification with antibody

- Block PVDF membrane for 1h in 1X TBS-T / 5% nonfat dried milk

  - Blocking solution (500ml):

    - 50 ml 10X TBS

    - 500 ul Tween

    - 25g nonfat dry milk powder

    - Add ddH2O to total volume 500 ml

- Incubate in primary antibody over night @ 4 celcius with lid

  - Primary antibody solution (can be reused ~5 times):

    - UBF (1:200): dilute UBF antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/200 = 150 ul antbody and the rest is diluent

    - C-MYC (1:200): dilute C-MYC antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/200 = 150 ul antbody and the rest is diluent

    - MAF (1:100): dilute MAF antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/100 = 300 ul antbody and the rest is diluent

    - MuRF (1:100): dilute MuRF antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/100 = 300 ul antbody and the rest is diluent

    - COL6A1 (1:100): dilute COL6A1 antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/100 = 300 ul antbody and the rest is diluent

- Wash 3 x 10 min in 1X TBS-T

  - 1X TBS-T solution (1 Liter):

    - 100 ml 10X TBS

    - 1 ml Tween

    - Add ddH2O to total volume 1000ml

- Incubate in Secondary antibody for 1h @ Room temperature (NB: Only needed for COL6A1)

  - Secondary antibody solution:

    - Mouse (1:10 000): dilute Mouse antibody in 1X TBS-T / 5% nonfat dried milk

For 1 whole PVDF membrane you need 30 ml.

Take 30ml/10000 =  3 ul antbody and the rest is diluent

- Wash 4 x 10 min in 1X TBS-T (3-6 x 5-10 min)

Enhanced Chemiluminesence:

- Mix 50/50 Luminol/peroxide for ECL for 5-7min

  - ~30ml for whole PVDF membrane (less if cut)

  - Can be reused intraday, but does not keep over night!

- Photograph with G:BOX on clear plastic sheet and black

- Dry @ RT untill completely dry

- Store in ziplock bag @ -20 celcius